Real-time safe driving score evaluation

Team STRL

User Manual

1. Introduction 🖥

This is a program that integrates the UC-win/Road driving simulator with a Python-based dashboard to analyze a driver's habits in real-time and evaluate their safe driving score.

Points are deducted when dangerous driving behaviors (such as speeding, sudden braking, etc.) occur, and the score is recovered for driving safely for more than five seconds. All score changes are visualized as a real-time graph in a separate window.

2. Development environment configuration ⚙

To use this system, the following programs and libraries must be installed on your computer.

- UC-win/Road: Driving simulation program

- VSCode: Code editor

- Python: Programming language (version 3.8 or higher recommended)

- Python Libraries: Open a terminal (CMD) and enter the following commands to install.

3. UC-win/Road setting ⚙️

To allow the Python dashboard to receive data, you must configure the data output settings in UC-win/Road.

1. After launching UC-win/Road, select ①Driving Simulation > ②Option Output from the top menu.

2. In the 'Log Output Settings' window, configure the Host[UDP] section.

- Click 'Add Host' to create a new item.

- IP Address: 127.0.0.1

- Port: 4000

3. In the 'Log Output Item Options' section, check the box for 'Driving Vehicle'.

4. Running the Python Dashboard 🚀

1. Save the source code on your computer as driving_dashboard.py

2. Open a terminal (CMD) and use the cd command to navigate to the folder where you saved the code.

3. Enter the command below to run the Python dashboard program.

4. If it runs successfully, a GUI window titled "Real-time Driving Score Dashboard" will appear.

5. Running the UC-win/Road Simulation 🚀

1. launch the UC-win/Road program.

2. From the top tab in UC-win/Road, click to start the 'Driving Simulation'.

3. Next, start the simulation and begin driving the vehicle.

4. As you drive, you can watch the score on the dashboard change and see deduction details recorded on the graph in real-time whenever a dangerous driving behavior occurs.

6. Dashboard Screen Description 📊

○ Driving Score: This is the current score, displayed at the top of the window. It starts at 100 points.

○ Graph:

- X-axis (horizontal): The simulation time at which a penalty or recovery event occurred.

- Y-axis (vertical): Driving scrore

- Red dots and line: Represents the trend of score changes over time.

- Text Labels: The cause of the score change, such as "Sudden Braking," "Speeding," or "Safe Driving (+3)," is displayed above the point where the score changed.